A F Fercher et al 2003 Rep. Prog. Phys. 66 239-303

## Optical coherence tomography - principles and applications

A F Fercher1, W Drexler1, C K Hitzenberger1 and T Lasser2
1 Institute of Medical Physics, University of Vienna, Waehringer Strasse 13, A-1090 Wien, Austria
2 Laboratoire d'optique biomédicale, Institut d'imagerie et optique appliquée, EPFL Lausanne, CH-1015 Ecublens, Switzerland

E-mail: adolf.friedrich.fercher@univie.ac.at

Received 23 October 2002
Published 20 January 2003 
Print publication: Issue 2 (February 2003)

Abstract. There have been three basic approaches to optical tomography since the early 1980s: diffraction tomography, diffuse optical tomography and optical coherence tomography (OCT). Optical techniques are of particular importance in the medical field, because these techniques promise to be safe and cheap and, in addition, offer a therapeutic potential. Advances in OCT technology have made it possible to apply OCT in a wide variety of applications but medical applications are still dominating. Specific advantages of OCT are its high depth and transversal resolution, the fact, that its depth resolution is decoupled from transverse resolution, high probing depth in scattering media, contact-free and non-invasive operation, and the possibility to create various function dependent image contrasting methods. This report presents the principles of OCT and the state of important OCT applications.

OCT synthesises cross-sectional images from a series of laterally adjacent depth-scans. At present OCT is used in three different fields of optical imaging, in macroscopic imaging of structures which can be seen by the naked eye or using weak magnifications, in microscopic imaging using magnifications up to the classical limit of microscopic resolution and in endoscopic imaging, using low and medium magnification. First, OCT techniques, like the reflectometry technique and the dual beam technique were based on time-domain low coherence interferometry depth-scans. Later, Fourier-domain techniques have been developed and led to new imaging schemes. Recently developed parallel OCT schemes eliminate the need for lateral scanning and, therefore, dramatically increase the imaging rate. These schemes use CCD cameras and CMOS detector arrays as photodetectors. Video-rate three-dimensional OCT pictures have been obtained. Modifying interference microscopy techniques has led to high-resolution optical coherence microscopy that achieved sub-micrometre resolution.

This report is concluded with a short presentation of important OCT applications. Ophthalmology is, due to the transparent ocular structures, still the main field of OCT application. The first commercial instrument too has been introduced for ophthalmic diagnostics (Carl Zeiss Meditec AG). Advances in using near-infrared light, however, opened the path for OCT imaging in strongly scattering tissues. Today, optical in vivo biopsy is one of the most challenging fields of OCT application. High resolution, high penetration depth, and its potential for functional imaging attribute to OCT an optical biopsy quality, which can be used to assess tissue and cell function and morphology in situ. OCT can already clarify the relevant architectural tissue morphology. For many diseases, however, including cancer in its early stages, higher resolution is necessary. New broad-bandwidth light sources, like photonic crystal fibres and superfluorescent fibre sources, and new contrasting techniques, give access to new sample properties and unmatched sensitivity and resolution.

doi:10.1088/0034-4885/66/2/204
URL: http://stacks.iop.org/0034-4885/66/239
PII: S0034-4885(03)18703-9

- PDF (1.84 MB)

- References

- Articles citing this article

Optical Coherence Tomography
Optical coherence tomography (OCT) and optical Doppler tomography (ODT) are noninvasive techniques that combine Doppler principle with coherence gating for tomographic imaging of in vivo tissue structure and blood flow, simultaneously. Our OCT/ODT system is fiber based with spatial resolution on the order of 5-10 µm and the velocity sensitivity on the order of 10 µm/s. Image speed for the current system is one second per frame. The high spatial resolution and velocity sensitivity allows noninvasive optical biopsy and functional imaging of changes in tumor structure and neovasculature.